IoT Based Smart Medicine Reminder and Monitoring System

Project Documentation

# Project Description

An IoT-based medicine reminder and dispenser simulation designed to improve medication adherence among elderly and chronically ill patients. An ESP32 handles reminders through a buzzer and OLED display and captures dose confirmation via a push-button, while an independent Arduino UNO rotates a servo motor to simulate dispensing at fixed intervals. The system works fully offline, with no internet or app required, making it low-cost and easy to deploy.

# Overview

Missing medication doses is a common problem, especially for elderly patients and those with chronic conditions. This project automates the reminder process and simulates automatic dispensing, while remaining low-cost, offline, and easy to use.

The system uses two independent microcontrollers:

- ESP32 - handles all user interaction: OLED display messages, buzzer alerts, and push-button confirmation.

- Arduino UNO - independently rotates a servo motor every 5 minutes to simulate pill dispensing.

Both boards run on their own non-blocking millis() timers and don't need to communicate with each other, which keeps the design simple and fault-tolerant.

# Features

- Automatic medicine reminders every 5 minutes (buzzer + OLED)

- Dose confirmation via push-button

- Second reminder retry if the dose isn't confirmed

- Simulated dispensing using a servo-driven rotating fan mechanism

- Fully offline - no Wi-Fi, app, or internet required

- Low-cost build (under Rs. 1500 in components)

# Hardware Used

| Component | Purpose |
| --- | --- |
| ESP32 Dev Module | User interface - display, buzzer, button |
| Arduino UNO | Servo motor control (dispensing) |
| SG90 Servo Motor | Rotates the dispenser mechanism |
| OLED Display (128x64, I2C) | Shows reminder / confirmation messages |
| Buzzer | Audio alert |
| Push Button | Dose confirmation input |
| Breadboard + Jumper Wires | Wiring |
| Power Bank / USB cable | Power supply |

# Software and Libraries

- Arduino IDE - https://www.arduino.cc/en/software

- ESP32 Board Package - https://github.com/espressif/arduino-esp32

- Adafruit GFX Library - https://github.com/adafruit/Adafruit-GFX-Library

- Adafruit SSD1306 Library - https://github.com/adafruit/Adafruit_SSD1306

- Arduino built-in Servo.h library

# How It Works

- Every 5 minutes, the ESP32 triggers a buzzer and OLED alert: "Time to take medicine!"

- The user presses the push button to confirm the dose. The display updates to "Dose taken!"

- If no confirmation is received, a second reminder is triggered after 1 minute.

- Independently, the Arduino UNO rotates the SG90 servo motor by 60 degrees every 5 minutes, simulating the release of one dose from the dispenser.

Since the two boards run on separate timers, there is no direct communication link between them - this keeps the design simple and avoids synchronization failures.

# Pin Connections

## ESP32

| Pin | Component |
| --- | --- |
| GPIO 26 | Buzzer |
| GPIO 4 | Push Button (INPUT_PULLUP) |
| SDA / SCL (default I2C pins) | OLED Display |

## Arduino UNO

| Pin | Component |
| --- | --- |
| D9 | Servo signal |
| D13 | Status LED |

# Project Structure

esp32/

sketch_esp32.ino     (ESP32 code - display, buzzer, button)

arduino_uno/

sketch_ard.ino       (Arduino UNO code - servo control)

These are the final, working versions of the firmware. Earlier drafts explored a single-board design and a wired-signal handoff between the two boards, but the versions used here run on fully independent timers on each board.

# Future Enhancements

- Cloud connectivity for dose logging and caregiver alerts (using ESP32 Wi-Fi)

- Companion mobile app

- Real-time clock (RTC) module for accurate real-world time tracking

- Voice-based reminders

- Battery backup for power-cut resilience

- Multi-dose / multi-user scheduling

- Sensor-based confirmation (load cell / IR) instead of a button

Setup Guide

This guide walks through installing everything needed to build and run the system, from software setup to wiring and uploading code.

## 1. Install the Arduino IDE

Download and install the Arduino IDE (version 2.x recommended): https://www.arduino.cc/en/software

Available for Windows, macOS, and Linux.

## 2. Add ESP32 Board Support

The ESP32 is not included by default in the Arduino IDE, so its board package needs to be added.

- Open Arduino IDE, go to File > Preferences

- In "Additional Board Manager URLs", paste: https://raw.githubusercontent.com/espressif/arduino-esp32/gh-pages/package_esp32_index.json

- Go to Tools > Board > Boards Manager

- Search for "esp32" and install the package by Espressif Systems

- Once installed, select the board: Tools > Board > ESP32 Arduino > ESP32 Dev Module

Reference: https://github.com/espressif/arduino-esp32

## 3. Install Required Libraries

Go to Sketch > Include Library > Manage Libraries, and install the following:

| Library | Used For | Source |
| --- | --- | --- |
| Adafruit SSD1306 | OLED display driver | github.com/adafruit/Adafruit_SSD1306 |
| Adafruit GFX Library | Graphics primitives for OLED | github.com/adafruit/Adafruit-GFX-Library |
| Servo | Controls the SG90 servo motor (usually pre-installed) | arduino.cc/reference/en/libraries/servo |

Search each name in the Library Manager search bar and click Install.

## 4. Install USB Drivers (if needed)

- ESP32 boards typically use a CP2102 or CH340 USB-to-serial chip. If the board is not detected, install the CP2102 driver (silabs.com) or CH340 driver (wch-ic.com).

- Arduino UNO boards generally work out of the box on Windows 10/11, macOS, and Linux without extra drivers. On older Windows versions, install the standard Arduino driver bundled with the IDE.

## 5. Wiring

ESP32:

| Component | ESP32 Pin |
| --- | --- |
| OLED SDA | SDA (default I2C, usually GPIO 21) |
| OLED SCL | SCL (default I2C, usually GPIO 22) |
| Buzzer (+) | GPIO 26 |
| Push Button | GPIO 4 (other leg to GND, uses internal pull-up) |

Arduino UNO:

| Component | UNO Pin |
| --- | --- |
| Servo signal wire | D9 |
| Status LED | D13 (onboard LED can be used) |

Power both boards via USB or a power bank.

## 6. Upload the Code

- Connect the ESP32 via USB, select the correct Port under Tools > Port, and select ESP32 Dev Module as the board.

- Open sketch_esp32.ino and click Upload.

- Disconnect, then connect the Arduino UNO, select Arduino Uno as the board and its Port.

- Open sketch_ard.ino and click Upload.

The two boards run independently - there is no direct wired communication between them, so they can be uploaded and powered separately.

## 7. Test the System

- OLED should show "Medicine Reminder" on boot.

- Every 5 minutes, the buzzer sounds and the OLED shows "Time to take medicine!"

- Press the push button to confirm - OLED should show "Dose taken!"

- The Arduino UNO's servo should rotate about 60 degrees every 5 minutes, independent of the ESP32.

## Troubleshooting

| Issue | Fix |
| --- | --- |
| "SSD1306 allocation failed" in Serial Monitor | Check I2C wiring (SDA/SCL) and confirm the OLED's I2C address (commonly 0x3C) |
| ESP32 not detected in Port list | Install CP2102/CH340 driver, try a different USB cable/port |
| Servo not moving | Check external power to the servo - USB power alone may be insufficient |
| Button not registering | Confirm wiring uses INPUT_PULLUP logic (button pulls pin to GND when pressed) |

## Reference Links

- Arduino millis() vs delay(): arduino.cc/en/Tutorial/BuiltInExamples/BlinkWithoutDelay

- ESP32 Technical Reference: espressif.com

- Arduino Servo Library Reference: arduino.cc/en/reference/servo

Appendix: Source Code

## ESP32 Code - sketch_esp32.ino

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

#define OLED_RESET -1

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, OLED_RESET);

#define BUZZER_PIN 26

#define BUTTON_PIN 4

unsigned long lastReminder = 0;

unsigned long alertStartTime = 0;

unsigned long lastBuzzToggleTime = 0;

unsigned long secondReminderStart = 0;

unsigned long doseConfirmationTime = 0;

const unsigned long reminderInterval = 5 * 60 * 1000; // 5 minutes

const unsigned long alertDuration = 10000; // 10 seconds

const unsigned long retryDelay = 60000; // 1 minute

bool buzzerState = false;

bool isAlerting = false;

bool stopAlert = false;

bool medicineTaken = false;

bool secondReminderScheduled = false;

bool showReminderText = false;

void setup() {

Serial.begin(115200);

pinMode(BUZZER_PIN, OUTPUT);

pinMode(BUTTON_PIN, INPUT_PULLUP);

if (!display.begin(SSD1306_SWITCHCAPVCC, 0x3C)) {

Serial.println(F("SSD1306 allocation failed"));

for (;;);

}

display.clearDisplay();

display.setTextSize(1);

display.setTextColor(WHITE);

display.setCursor(0, 10);

display.print("Medicine Reminder");

display.display();

delay(2000);

}

void loop() {

unsigned long currentTime = millis();

// If it's time for the next scheduled dose

if (currentTime - lastReminder >= reminderInterval) {

// Reset all flags

medicineTaken = false;

stopAlert = false;

secondReminderScheduled = false;

isAlerting = false;

triggerReminder();

lastReminder = currentTime;

}

// Background buzzer handler

handleAlert();

// Retry second reminder after 1 minute if dose not confirmed

if (!medicineTaken && !isAlerting && secondReminderScheduled && (currentTime - secondReminderStart >= retryDelay)) {

triggerReminder();  // Second alert

secondReminderScheduled = false;

}

// Button press check

if (digitalRead(BUTTON_PIN) == LOW && !medicineTaken) {

stopAlert = true;

medicineTaken = true;

isAlerting = false;

secondReminderScheduled = false;

confirmDose();

}

if (showReminderText && millis() - doseConfirmationTime >= 10000) {

display.clearDisplay();

display.setCursor(0, 10);

display.print("Medicine Reminder");

display.display();

showReminderText = false;

}

}

void triggerReminder() {

stopAlert = false;

isAlerting = true;

alertStartTime = millis();

lastBuzzToggleTime = 0;

buzzerState = false;

displayMessage();

}

void displayMessage() {

display.clearDisplay();

display.setCursor(0, 10);

display.print("Time to take medicine!");

display.display();

}

void confirmDose() {

stopAlert = true;

isAlerting = false;

noTone(BUZZER_PIN); // FORCE buzzer off

display.clearDisplay();

display.setCursor(0, 10);

display.print("Dose taken!");

display.display();

doseConfirmationTime = millis();  // Save the time

showReminderText = true;

}

void handleAlert() {

if (!isAlerting) return;

unsigned long currentTime = millis();

if (stopAlert || currentTime - alertStartTime >= alertDuration) {

noTone(BUZZER_PIN);

isAlerting = false;

if (!medicineTaken && !secondReminderScheduled) {

secondReminderStart = millis();

secondReminderScheduled = true;

}

return;

}

if (currentTime - lastBuzzToggleTime >= 300) {

if (buzzerState) {

noTone(BUZZER_PIN);

} else {

tone(BUZZER_PIN, 1000);

}

buzzerState = !buzzerState;

lastBuzzToggleTime = currentTime;

}

}

## Arduino UNO Code - sketch_ard.ino

#include <Servo.h>

Servo myServo;

int angle = 0;

unsigned long previousMillis = 0;

const unsigned long interval = 5 * 60 * 1000UL; // 5 minutes in milliseconds

void setup() {

myServo.attach(9);         // Servo signal pin connected to D9

myServo.write(0);          // Start at 0 degrees

pinMode(13, OUTPUT);       // Set pin 13 as output for LED

digitalWrite(13, HIGH);    // Keep LED ON

}

void loop() {

unsigned long currentMillis = millis();

// Rotate servo every 5 minutes

if (currentMillis - previousMillis >= interval) {

previousMillis = currentMillis;

angle += 60;

if (angle > 180) {

angle = 0;  // Reset to 0 after full sweep

}

myServo.write(angle);

}

}